	*** IMPORTANT WARRANTY INFORMATION ***

	
	
	

	
	Spindle Cable and Air Purge Unit
	

	
	
	

	
	All DTS spindles require an air purge for proper operation.  Clean, dry, non-lubricated air is to be supplied by the end-user to the air purge system.  The air pressure should be adjusted to read approximately 15 psi.  Air exits the air purge unit and flows through the nylon tubing and into the spindle cable.  Air continues through the spindle cable, into the spindle near the motor, through the spindle bearings, and finally exits through the seal at the spindle face.  Proper installation and use of the air purge unit ensures that contaminants do not enter the spindle and assists in prolonging the life of the spindle.  Please note that since the spindle cable contains an air tube, the bend radius of the spindle cable should not be less than 2" at any time.
	

	
	
	

	
	
[image: image1.wmf]
	

	
	
	

	
	[image: image2.jpg]solutions





	
	
	
	

	
	
	G II Solutions, Inc.
	Phone:
	864.639.6605
	

	
	
	201 Bank Street
	Fax:
	864.639.4056
	

	
	
	P.O. Box 428
	E-mail:
	info@giisolutions.com
	

	
	
	Central, SC 29630
	Web Site:
	www.giisolutions.com
	

	
	
	
	
	

	
	
	

	*** IMPORTANT WARRANTY INFORMATION ***


Air purge should be continuous for an application where the spindle is mounted vertically with the spindle nose pointing up.








_1167554761.dwg

